An ATV gives you an unparalleled thrill and adrenalin rush, and, understandably, you want to ride your vehicle throughout the year.  While snow and cold temperatures scare most ATV enthusiasts, you can still explore the outdoors safely if you prep your vehicle and equip yourself for these treacherous conditions. Here are some suggestions for preparing yourself for a safe ATV ride in cold weather and snowy conditions.

Prep the ATV for Snow

The following preparatory steps are necessary for safe ATV riding in winter.

- Change the oil: If your ATV is due for an oil change, take the opportunity to do this before winter. Use a light synthetic oil that will flow quickly throughout the engine and keep it well lubricated.

- Fit tire chains: Riding in extremely snowy conditions is dangerous due to poor traction. Tire chains improve traction in these wet conditions allowing you to enjoy your ride.

- Change the plugs: For an older ATV, changing the plugs can prevent hard starts, and it will keep the machine running in low temperatures.

- Power maintenance: Clean the battery terminals to clear any corrosion and to avoid power problems, connect a permanent trickle charger for the battery to charge overnight.

- Fuel stabilizer: In winter, it is easy for moisture to seep into your ATV’s fuel tank leading to hard starts. You can overcome this problem by adding a fuel stabilizer in the fuel tank.

- Install ATV winter accessories: Some accessories such as wind-deflecting handlebars, snowplows, winches, track kits heated grips, seat warmer, and thumb warmers can improve your ride in the cold, snowy conditions.

Layer Your Clothing

Riding in freezing temperatures is tough, and you need to find the warmest clothing to keep your body temperatures stable. Layering is a smart idea to beat the cold. The first layer of wool or wool/synthetic clothing wicks away the sweat to prevent chilling. Next comes an insulating fleece vest or pullover to trap your body heat. The third/outer layer of clothing is a waterproof/breathable shell to cut the wind and keep you dry.

Riding Gear

In addition to your clothing, you need the right gear to enjoy your ATV in winter. Essential accessories include insulated gloves and thick wool socks, winter boots, winter goggles, facemask or a balaclava and a helmet.

A thorough inspection of your ATV is a great idea before the cold weather sets in, and if you are in Michigan Center, MI or the surrounding areas, we are glad to help. TC Powersports is a one-stop-shop for all your motorcycle, scooter, ATV and UTV needs including sales, servicing, parts and accessories, and winter storage. Contact us to schedule an appointment today and let’s help prep that ATV for winter riding.