4 Innovative Techniques Used in Trenchless Sewer Repairs

A sewer line is an essential part of your home’s drainage system, but unlike other drainage the section is out of sight. Most homeowners forget about the essential drainage component until sewage backflow problems emerge. A failed sewage drainage system also causes a stench in your home, increases the risk of disease, and property damage. One way to resolve the problem is through trenchless sewer repair. The innovative plumbing technology requires no surface excavation for the drainage system repairs.

Through the less invasive plumbing technology, you save money and time during sewage drainage system maintenance. Here are the innovative techniques used in the advanced plumbing repair.

- Slip Lining Repair

Slip lining is a trenchless plumbing repair technique in which your plumber pulls a smaller but durable polyethylene carrier pipe into the larger damaged host pipe. The technique replaces the damaged section of the older pipe. Once in place, the contractor seals the space between the two pipes with grout. The sewer repair procedure saves time and money, and minimizes damage to your home’s landscaping.

- Spray Lining/ Brush Coating Repair

Plumbers also use spray lining or brush coating in pipe repairs. The plumber sprays or brushes epoxy or another resin onto the interior of the damaged pipe to resolve problems such as corrosion and leaky joints. The resin coat on the pipes not only repairs corrosion or leaks but also increases the life of the sewer line.

Plumbing contractors use the spraying or coating technique for cast iron pipes which are prone to rust and to eliminate the risk of toxic fumes in your house. Sewer gas is one of the main safety hazards associated with leaky joints in sewage drains.

- Cured-in-Place-Pipe Repair

Cracks are one of the most common forms of damage in drainage pipes. The pressure of drain water and other environmental factors increases the risk of cracks in the sewer line. Traditional plumbing cracks repair involved extensive excavation around the site of the leak. Today, CIPP technology offers a more efficient solution to the problem through internal pipe repair.

In the advanced form of plumbing repair, a plumber creates a new pipe inside the old one. The structural repair procedure involves a fiberglass absorbent tube driven into the older pipe and attached with a two-part epoxy resin.

The liner cures in place to solve any leak problems in the sewage drainage system plumbing. Plumbers speed up the process with UV light or steam to create a permanent liner on the damaged pipe. The process is minimally invasive plumbing repair method, which lowers the time and cost of sewer repair. The rehabilitated pipe also lasts a long time.

- Pipe Bursting Repair

If you have a damaged sewer line, pipe bursting is a cheaper alternative to traditional trench excavation for repairs. The innovative plumbing repair technology involves the use of a replacement pipe that’s pulled through the old damaged line through a winch. Plumbing contractors use a bursting head to guide the replacement pipe made of high-density polyethylene (HDPE) through the older pipe.

The technology works perfectly for replacement of damaged lateral lines which connect your sewer line to the public main. The sewer repair technique is cheaper, faster, and you get a more durable pipe. With the technology, you have can install a bigger drainage line to suit your home’s drainage needs.

A damaged sewer line poses a great risk to your family and your house. The sewage backflow caused by drainage problems causes property damage and exposes the family to disease. If you have a sewer problem in Seattle, WA, or the surrounding areas, contact us today at Aurora Plumbing and Electric Supply, Inc for excellent plumbing solutions.

Sources:

https://www.iii.org/article/protect-your-house-from-sewer-backups

https://www.sciencedirect.com/topics/engineering/trenchless-technology

https://www.ncbi.nlm.nih.gov/pmc/articles/PMC1275984/

https://keepsafeguide.enterprisecommunity.org/en/prevent-wastewater-backflow-homes

https://www.businessinsider.com/personal-finance/does-homeowners-insurance-cover-sewer-line-damage-replacement?r=US&IR=T